import serial

import time

import csv

# =========================================

# SETTINGS

# =========================================

PORT = "COM3"

BAUDRATE = 115200

RECORD_TIME = 60   # 60 seconds

# =========================================

# CONNECT TO ARDUINO

# =========================================

print("======================================")

print("       MPU6050 DATA RECORDER")

print("======================================")

print()

print("Connecting to Arduino on", PORT)

try:

ser = serial.Serial(

PORT,

BAUDRATE,

timeout=1

)

except serial.SerialException as e:

print()

print("ERROR: Could not connect to Arduino.")

print(e)

print()

print("Check:")

print("1. Arduino is connected")

print("2. Port is COM3")

print("3. Arduino Serial Monitor is closed")

input("Press Enter to exit...")

exit()

# Arduino reset after serial connection

time.sleep(2)

# Clear old data waiting in serial buffer

ser.reset_input_buffer()

print("Arduino connected!")

print()

# =========================================

# COUNTDOWN

# =========================================

print("Recording will start in 3 seconds...")

print()

for i in range(3, 0, -1):

print(i)

time.sleep(1)

print()

print("======================================")

print("         RECORDING STARTED")

print("======================================")

print()

print("Move the MPU6050 sensor.")

print("All readings will be shown below.")

print()

print("CSV is being created automatically.")

print()

# =========================================

# CREATE CSV FILE

# =========================================

with open(

"mpu6050_data.csv",

"w",

newline=""

) as file:

writer = csv.writer(file)

# CSV header

writer.writerow([

"Timestamp_ms",

"ax",

"ay",

"az",

"gx",

"gy",

"gz"

])

# =====================================

# START TIMER

# =====================================

start_time = time.time()

sample_count = 0

# =====================================

# RECORD DATA

# =====================================

while time.time() - start_time < RECORD_TIME:

line = ser.readline().decode(

"utf-8",

errors="ignore"

).strip()

if not line:

continue

parts = line.split(",")

# Arduino should send 7 values

if len(parts) != 7:

continue

try:

# Convert values to numbers

values = [int(value) for value in parts]

except ValueError:

# Ignore invalid lines

continue

# =================================

# SHOW READING IN TERMINAL

# =================================

print(

f"Time: {values[0]} ms | "

f"ax: {values[1]} | "

f"ay: {values[2]} | "

f"az: {values[3]} | "

f"gx: {values[4]} | "

f"gy: {values[5]} | "

f"gz: {values[6]}"

)

# =================================

# SAVE READING TO CSV

# =================================

writer.writerow(values)

# Make sure data is written immediately

file.flush()

sample_count += 1

# =========================================

# CLOSE SERIAL

# =========================================

ser.close()

# =========================================

# FINISHED

# =========================================

print()

print()

print("======================================")

print("        RECORDING COMPLETE")

print("======================================")

print()

print("Total readings recorded:", sample_count)

print()

print("CSV file created:")

print("mpu6050_data.csv")

print()

print("The CSV is saved in the same folder")

print("as record.py.")

print()

input("Press Enter to exit...")