import numpy as np

import pandas as pd

import matplotlib.pyplot as plt

from scipy.integrate import cumulative_trapezoid

# ============================================================

# SETTINGS

# ============================================================

INPUT_FILE = "mpu6050_data.csv"

PROCESSED_FILE = "mpu6050_processed.csv"

ACCELERATION_GRAPH = "acceleration_graph.png"

VELOCITY_GRAPH = "velocity_graph.png"

DISPLACEMENT_GRAPH = "displacement_graph.png"

# MPU6050 ±2g setting

LSB_PER_G = 16384.0

# Gravity

G = 9.80665

# ============================================================

# STEP 1 — LOAD CSV

# ============================================================

print("Loading CSV...")

try:

df = pd.read_csv(INPUT_FILE)

except FileNotFoundError:

print()

print("ERROR: mpu6050_data.csv not found!")

print("Make sure analyze.py and mpu6050_data.csv")

print("are in the same folder.")

input("Press Enter to exit...")

exit()

except pd.errors.EmptyDataError:

print()

print("ERROR: The CSV file is empty!")

input("Press Enter to exit...")

exit()

print("CSV loaded successfully!")

print("Number of readings:", len(df))

# ============================================================

# STEP 2 — CHECK REQUIRED COLUMNS

# ============================================================

required_columns = [

"Timestamp_ms",

"ax",

"ay",

"az"

]

for column in required_columns:

if column not in df.columns:

print()

print("ERROR: Missing column:", column)

input("Press Enter to exit...")

exit()

# ============================================================

# STEP 3 — TIME

# ============================================================

time = df["Timestamp_ms"].to_numpy(dtype=float)

# Convert milliseconds → seconds

time = time / 1000.0

# Start time from zero

time = time - time[0]

# ============================================================

# STEP 4 — RAW ACCELEROMETER DATA

# ============================================================

ax_raw = df["ax"].to_numpy(dtype=float)

ay_raw = df["ay"].to_numpy(dtype=float)

az_raw = df["az"].to_numpy(dtype=float)

# ============================================================

# STEP 5 — RAW → m/s²

# ============================================================

print()

print("Converting acceleration...")

ax = (ax_raw / LSB_PER_G) * G

ay = (ay_raw / LSB_PER_G) * G

az = (az_raw / LSB_PER_G) * G

# ============================================================

# STEP 6 — INITIAL CALIBRATION

# ============================================================

print("Calculating initial bias...")

# First 5 seconds are used for calibration

calibration_time = 5.0

calibration_indices = time <= calibration_time

# Safety check

if np.sum(calibration_indices) < 5:

print("WARNING: Not enough data for 5-second calibration.")

calibration_indices = np.arange(

min(50, len(time))

)

# Calculate initial bias

ax_bias = np.mean(

ax[calibration_indices]

)

ay_bias = np.mean(

ay[calibration_indices]

)

az_bias = np.mean(

az[calibration_indices]

)

# Remove initial bias

ax_linear = ax - ax_bias

ay_linear = ay - ay_bias

az_linear = az - az_bias

# ============================================================

# STEP 7 — VELOCITY

# ============================================================

print("Calculating velocity...")

vx = cumulative_trapezoid(

ax_linear,

time,

initial=0

)

vy = cumulative_trapezoid(

ay_linear,

time,

initial=0

)

vz = cumulative_trapezoid(

az_linear,

time,

initial=0

)

# ============================================================

# STEP 8 — DISPLACEMENT

# ============================================================

print("Calculating displacement...")

sx = cumulative_trapezoid(

vx,

time,

initial=0

)

sy = cumulative_trapezoid(

vy,

time,

initial=0

)

sz = cumulative_trapezoid(

vz,

time,

initial=0

)

# ============================================================

# STEP 9 — SAVE PROCESSED CSV

# ============================================================

print()

print("Saving processed CSV...")

df["Acceleration_X_mps2"] = ax_linear

df["Acceleration_Y_mps2"] = ay_linear

df["Acceleration_Z_mps2"] = az_linear

df["Velocity_X_mps"] = vx

df["Velocity_Y_mps"] = vy

df["Velocity_Z_mps"] = vz

df["Displacement_X_m"] = sx

df["Displacement_Y_m"] = sy

df["Displacement_Z_m"] = sz

df.to_csv(

PROCESSED_FILE,

index=False

)

print("Saved:", PROCESSED_FILE)

# ============================================================

# STEP 10 — ACCELERATION GRAPH

# ============================================================

print()

print("Creating acceleration graph...")

plt.figure(figsize=(12, 6))

plt.plot(

time,

ax_linear,

label="X"

)

plt.plot(

time,

ay_linear,

label="Y"

)

plt.plot(

time,

az_linear,

label="Z"

)

plt.title(

"MPU6050 Acceleration vs Time"

)

plt.xlabel(

"Time (seconds)"

)

plt.ylabel(

"Acceleration (m/s²)"

)

plt.legend()

plt.grid(True)

plt.tight_layout()

# Save image

plt.savefig(

ACCELERATION_GRAPH,

dpi=300,

bbox_inches="tight"

)

print("Saved:", ACCELERATION_GRAPH)

# ============================================================

# STEP 11 — VELOCITY GRAPH

# ============================================================

print("Creating velocity graph...")

plt.figure(figsize=(12, 6))

plt.plot(

time,

vx,

label="X"

)

plt.plot(

time,

vy,

label="Y"

)

plt.plot(

time,

vz,

label="Z"

)

plt.title(

"MPU6050 Velocity vs Time"

)

plt.xlabel(

"Time (seconds)"

)

plt.ylabel(

"Velocity (m/s)"

)

plt.legend()

plt.grid(True)

plt.tight_layout()

# Save image

plt.savefig(

VELOCITY_GRAPH,

dpi=300,

bbox_inches="tight"

)

print("Saved:", VELOCITY_GRAPH)

# ============================================================

# STEP 12 — DISPLACEMENT GRAPH

# ============================================================

print("Creating displacement graph...")

plt.figure(figsize=(12, 6))

plt.plot(

time,

sx,

label="X"

)

plt.plot(

time,

sy,

label="Y"

)

plt.plot(

time,

sz,

label="Z"

)

plt.title(

"MPU6050 Displacement vs Time"

)

plt.xlabel(

"Time (seconds)"

)

plt.ylabel(

"Displacement (m)"

)

plt.legend()

plt.grid(True)

plt.tight_layout()

# Save image

plt.savefig(

DISPLACEMENT_GRAPH,

dpi=300,

bbox_inches="tight"

)

print("Saved:", DISPLACEMENT_GRAPH)

# ============================================================

# STEP 13 — SHOW ALL THREE GRAPHS

# ============================================================

print()

print("Opening graphs...")

plt.show()

# ============================================================

# FINISHED

# ============================================================

print()

print("==============================================")

print("              ANALYSIS COMPLETE")

print("==============================================")

print()

print("Files created:")

print("1. acceleration_graph.png")

print("2. velocity_graph.png")

print("3. displacement_graph.png")

print("4. mpu6050_processed.csv")

print()

print("All files are saved in the same folder.")

input("Press Enter to exit...")